5.2.2. INQUIRY Command

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Operation Code (12h) |  |  |  |  |  |  |  |
| 1 | Logical Unit Number |  |  | Reserved |  |  |  |  |
| 2 | Reserved |  |  |  |  |  |  |  |
| 3 | Reserved |  |  |  |  |  |  |  |
| 4 | Allocation Length |  |  |  |  |  |  |  |
| 5 | Control Byte |  |  |  |  |  |  |  |

The INQUIRY command requests that information regarding parameters of the controller and its attached peripheral device be sent to the initiator.

The INQUIRY command will be terminated with a BUSY status when the controller is performing power-on diagnostics or reading of the table of contents is occurring.

The INQUIRY command will be terminated with a CHECK CONDITION status only then the controller has detected a catastrophic error.

If an INQUIRY command is received from an initiator with a pending unit attention condition (before the controller reports CHECK CONDITION status), the controller will perform the INQUIRY command and will not clear the unit attention condition.

Table 5-1:  Inquiry Data

| Bit Byte | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Peripheral Device Type (05h) |  |  |  |  |  |  |  |
| 1 | RMB | Device Type Qualifier |  |  |  |  |  |  |
| 2 | ISO version |  | ECMA version |  |  | ANSI-Approved version |  |  |
| 3 | Reserved |  |  |  | Response Data Format |  |  |  |
| 4 | Allocation Length(33h) |  |  |  |  |  |  |  |
| 5 | Reserved |  |  |  |  |  |  |  |
| 6 | Reserved |  |  |  |  |  |  |  |
| 7 | Rel-Add | WB32 | WB16 | Sync | Link | Rsrvd | CmdQ | SRST |
| 8 | (MSB) Vendor Identification (LSB) |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |
| 16 | (MSB) Product Identification (LSB) |  |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  |  |
| 32 | (MSB) Product Revision Level (LSB) |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |  |
| 36 | (MSB) Firmware Date and Time(in ASCII) (LSB) |  |  |  |  |  |  |  |
| 55 |  |  |  |  |  |  |  |  |

The peripheral device type code is set to 05h if the logical unit is present. If the logical unit is not present the peripheral device type code is set to 7Fh.

The RMB bit is set to one because the disc is removable.

The device-type qualifier is a set to zero.

The ANSI-Approved Version is set to 02h.

The response data format is set to 02h.

The additional length is set to 33h. If the allocation length of the command  descriptor block is too small to transfer all of the parameters, the additional length will not be adjusted to reflect the truncation.

The Sync bit of one indicates that the rive supports synchronous data transfer.

The Link bit of one indicates that the drive supports linked command.

The relative addressing (RelAdr) bit of one indicates that the device supports the relative addressing mode for this logical unit.  If this bit is set to one, the linked command (Linked) bit shall be set to one; since relative addressing can only be used with linked commands.  A RelAdr bit of zero indicates the device does not support relative addressing for this logical unit.

A wide bus 32 (Wbus32) bit of one indicates that the device supports 32-bit wide data transfers. A value of zero indicates that the device does not support 32-bit wide data transfers.

A wide bus 16 (Wbus16) bit of one indicates that the devices supports 16-bit wide data transfers.  A value of zero indicates that the devices does not support 16-bit wide data transfers.

NOTE: If the value of both the Wbus16 and Wbus32 bits are zero, the device only supports 8-bit wide data transfers.

A synchronous transfer (Sync) bit of one indicates that the devices supports synchronous data transfer.  A value of zero indicates the device does not support synchronous data transfer.

A linked command (Linked) bit of one indicates that the device supports linked commands for logical unit.  A value of zero indicates the device does not support linked commands for this logical unit.

A command queuing (CmdQue) bit of one indicates that the device supports tagged command queuing for this logical unit.  A value of zero indicates the device does not support tagged command queuing for this logical unit.

A soft reset (SftRe) bit of zero indicates that the device responds to the RESET condition with the hard RESET alternative.  A SftRe bit of one indicates that the device responds to the RESET condition with the soft RESET alternative.

The vendor identification field contains eight bytes of ASCII data identifying the vendor of the product as follows:

| Byte | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | P | L | E | X | T | O | R | sp* |
| Code | 50h | 4Ch | 45h | 58h | 54h | 4Fh | 52h | 20h |

* ASCII space

The product identification field for the PX-4XCH contains sixteen bytes of ASCII data defined as follows:

| Byte | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | C | D | - | R | O | M | sp | P | X | - | 4 | X | C | H | sp | sp |
| Code | 43h | 44h | 2Dh | 52h | 4Fh | 4Dh | 20h | 50h | 58h | 2Dh | 34h | 58h | 43h | 48h | 20h | 20h |

The product identification field for the PX-4XCS contains sixteen bytes of ASCII data defined as follows:

| Byte | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | C | D | - | R | O | M | sp | P | X | - | 4 | X | C | S | sp | sp |
| Code | 43h | 44h | 2Dh | 52h | 4Fh | 4Dh | 20h | 50h | 58h | 2Dh | 34h | 58h | 43h | 53h | 20h | 20h |

The product identification field for the PX-4XCE contains sixteen bytes of ASCII data defined as follows:

| Byte | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | C | D | - | R | O | M | sp | P | X | - | 4 | X | C | E | sp | sp |
| Code | 43h | 44h | 2Dh | 52h | 4Fh | 4Dh | 20h | 50h | 58h | 2Dh | 34h | 58h | 43h | 45h | 20h | 20h |

The product identification field for the PX-6XCS contains sixteen bytes of ASCII data defined as follows:

| Byte | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | C | D | - | R | O | M | sp | P | X | - | 6 | X | C | S | sp | sp |
| Code | 43h | 44h | 2Dh | 52h | 4Fh | 4Dh | 20h | 50h | 58h | 2Dh | 36h | 58h | 43h | 53h | 20h | 20h |

The product identification field for the PX-8XCS contains sixteen bytes of ASCII data defined as follows:

| Byte | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | C | D | - | R | O | M | sp | P | X | - | 8 | X | C | S | sp | sp |
| Code | 43h | 44h | 2Dh | 52h | 4Fh | 4Dh | 20h | 50h | 58h | 2Dh | 38h | 58h | 43h | 53h | 20h | 20h |

The product identification field for the PX-12XSx contains sixteen bytes of ASCII data defined as follows:

| Byte | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | C | D | - | R | O | M | sp | P | X | - | 1 | 2 | (X) | S | sp | sp |
| Code | 43h | 44h | 2Dh | 52h | 4Fh | 4Dh | 20h | 50h | 58h | 2Dh | 31h | 32h | xxh | 53h | 20h | 20h |

Byte 28 data of PX-12XSx is PX-12CSx = “C”(code=43h), PX-12TSx = “T”(code=54h).

The product identification field for the PX-20TSx contains sixteen bytes of ASCII data defined as follows:

| Byte | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | C | D | - | R | O | M | sp | P | X | - | 2 | 0 | T | S | sp | sp |
| Code | 43h | 44h | 2Dh | 52h | 4Fh | 4Dh | 20h | 50h | 58h | 2Dh | 32h | 30h | 54h | 53h | 20h | 20h |

The product identification field for the PX-32XSx contains sixteen bytes of ASCII data defined as follows:

| Byte | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | C | D | - | R | O | M | sp | P | X | - | 3 | 2 | (X) | S | sp | sp |
| Code | 43h | 44h | 2Dh | 52h | 4Fh | 4Dh | 20h | 50h | 58h | 2Dh | 33h | 32h | xxh | 53h | 20h | 20h |

Byte 28 data of PX-32XSx is PX-32TSx = “T”(code=54h), PX-32CSx = “C”(code=43h).

The product identification field for the PX-40TSx contains sixteen bytes of ASCII data defined as follows:

| Byte | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASCII | C | D | - | R | O | M | sp | P | X | - | 4 | 0 | T | S | sp | sp |
| Code | 43h | 44h | 2Dh | 52h | 4Fh | 4Dh | 20h | 50h | 58h | 2Dh | 34h | 30h | 54h | 53h | 20h | 20h |

The product revision level field contains four bytes of ASCII data which indicate the revision level of the controller firmware defined as follows:

| Byte | 32 | 33 | 34 | 35 |
| --- | --- | --- | --- | --- |
| ASCII | (X) | . | (X) | (X) |
| Code | 3xh | 2Eh | 3xh | 3xh |

Byte 32 - A value of three indicates that this is a production release of the firmware. A value of two indicates a beta test version of the firmware. A value of one indicates an Engineering development version of the firmware.

Byte 33 - is a period.

Byte 34 - This byte indicates major revisions of, or additions to the firmware. The value of this byte is from zero to nine. This always implies a change in the implementation documentation.

Byte 35 - This byte indicates minor changes to the firmware, i.e. bug fixes, corrections for misunderstandings, etc. There is usually no change required, or very minor change in the implementation document.