Annex J CD-TEXT Format in the Lead-in Area (informative)

This annex explains the CD-TEXT information that is stored in the Lead-in Area as raw R-W Sub-channel data. The information here is stored in a memory and can be retrieved to the Initiator immediately.

J.1. General

The CD-TEXT information in the Lead-in area is retrieved from raw R-W Sub-channel data. The data format of RAW Sub-channel is explained in sub-clause 6.1.16. READ CD Command. 6 bits of each byte are R-W Raw data and it is converted from 6 bits to 8 bits from the 1st bytes, it makes 4 chunk of 18 bytes data. Each 18 bytes of data is called CD-TEXT Pack Data as shown in Table J.1. CD-TEXT information is recorded repeatedly in the Lead-in area and this one repeated data is called Text Group. Text Group consists of up to 8 types of language Blocks. Each Block represents one language and consists of maximum 255 Pack Data.

Table J.1. defines the contents of one Pack Data.

Pack is used to indicate the chunk of data in 18 bytes as shown above. Each Pack consists of a Header Field, Text Data Field and CRC Field.

NOTE from Jon: Above table is wrong!  Byte 1 = Track Number, Byte 2 = Seq number

Pack Type Indicator has the value and descriptions defined in Table J.2. Packs shall be encoded in the order of the items listed in the Table.

Track Number Indicator consists with 2 information. MSB of this byte in the Extension Flag and is normally set to 0b. If it is set to 1b, the Pack is used for an extended application (TBD). The rest of the byte is used for Track Number or Pack Element Number. Track Number is used when the Text Data field belongs to the track.

If the Pack is independent of the Track, this field indicates Pack Element Number that depends on the type of the Pack.

Sequence Number Indicator is the number incrementally increased from the first Pack to the end in each Block. It starts from 00h to FFh.

Block Number and Character Position Indicator contains 3 fields of information defined in the following figure.

Bit 7(MSB) is the Double Byte Character Code indication, that indicates if Text Data Field contains the Double Byte Character Code or not. If it is set to 0b, the Single Byte Character Code is used.

Bit 4 to 6, 3 bits, indicate the Block Number of the Block to that the Pack belongs. A Block is used to indicate a set of text information representing one particular language. It can be used up to 8 at the same time.

Bit 0 to Bit 3, indicate the Character Position. It is the number of character in the strings that belongs to the Text Data Field in the previous Pack. The Character Position starts from 0 to 15 and 15 indicates that the first character belongs to the one before the previous Pack. When the character code is double byte code, a set of 2 bytes in the Text Data Field is counted at one.

A null code is also counted as a character, that indicates termination of each strings.

Character Position is not used in Packs with ID1=88h, 89h and 8Fh. 00h shall be used in all these Packs.

A Text Data Field consists of 12 bytes. It contains either character strings or binary information depending on the type of Pack. All data in this field shall be transferred as recorded on the disc.

Packs except ID1=88h, 89h and 8Fh shall contains character strings in the Text Data Field. If Packs with ID1=80h to 85h, and 8Eh are used, a character strings for each track shall be provided.

A character string consists of series of characters and a terminator(One null code for single byte, two null code for double byte)`

The size of a character string is recommended to be less than 160 bytes. If a character string does not fit in a Text Data Field of a Pack, it is continued onto the succeeding Packs. The succeeding character string will be encoded starting at the next byte in the Text Data Field after the terminator of the current string. Unused bytes in the Text Data Field shall be filled with null codes.

In case the same character strings is used for consecutive tracks, the Tab Indicator may be used to indicate the same as previous track. It is a single tab code (09h) for single byte codes, and two tab codes for double bytes character codes. It shall not used for the first track.

Packs with ID1=86h, 87h, 88h, 89h and 8Fh contains binary information in the Text Data Field.

CRC Field consists of 2 bytes. Initiator system may use these bytes to check errors in the Pack. The polynomial is X16 + X12 + X5 + 1. All bits shall be inverted. This field is not mandatory for supporting CD-TEXT data.